I need help to change the following C code to work on ARM.

- Here is the code that is being used in a RX interrupt.

Enable Code.

pie1.RCIE=1;

intcon.PEIE = 1;

set_bit(rcsta, CREN);

intcon.GIE = 1;

Handler Code.

if(pir1 & (1 <<RCIF))

{

FCM_RS232_Rx();

clear_bit(pir1, RCIF);

}

- Also have some C code in the project as follows.

CV_RX_BUFF_START++;

if(FCV_RX_BUFF_START == COMMS_BUFF_SIZE)

FCV_RX_BUFF_START = 0;

- More C Code

char dummy;

if ((rcsta & (1 << FERR)) != 0)

{

dummy = rcreg;      //need to read the rcreg to clear FERR

}

else if ((rcsta & (1 << OERR)) != 0)

{

//need to read the rcreg to clear error

clear_bit(rcsta, CREN);

set_bit(rcsta, CREN);

}

else {

FCV_RX_BUFF[FCV_RX_BUFF_END] = rcreg; //no error, so rx byte is valid

FCV_RX_BUFF_END++;

if(FCV_RX_BUFF_END == COMMS_BUFF_SIZE)

FCV_RX_BUFF_END = 0;

}